# ECEN 100 Project: Proximity Sensing Project

# Introduction

Another of the sensors in your kit is the PIR sensor. This can sense the presence of objects or motion in the vicinity.

HC-SR501 Sensor is the sensor you’ll use for this project.

# Hardware Needed

You’ll need the HC-SR501 Sensor from your kit.

# Connecting to the HC-SR501 Sensor

You’ll only need three wires to connect to the PIR sensor. GND, VCC, and the signal.

There are additional adjustments on the PIP sensor that you need to be aware of. Here is the back of the sensor.

# Initial Arduino Sketch to Access the HC-SR501

Here is some initial code

// Define pins

const int pirPin = 13;    // PIR sensor output pin

const int ledPin = 6;   // Built-in Arduino LED

void setup() {

pinMode(pirPin, INPUT);  // Set PIR pin as input

pinMode(ledPin, OUTPUT); // Set LED pin as output

Serial.begin(115200);

Serial.println("Warming up sensor...");

// The HC-SR501 needs about 30-60 seconds to "settle"

// and learn the ambient infrared signature of the room.

delay(30000);

Serial.println("Sensor Active");

}

void loop() {

int motionState = digitalRead(pirPin); // Read the sensor

if (motionState == HIGH) {

digitalWrite(ledPin, HIGH);  // Turn LED ON

Serial.println("Motion Detected!");

} else {

digitalWrite(ledPin, LOW);   // Turn LED OFF

Serial.println("Scanning...");

}

delay(500); // Small delay to avoid flooding the serial monitor

}

When you upload and run the code you should see the led light when motion is detected. If you watch the Serial Monitor you’ll also see additional information.

Important to note is that this sketch allows the sensor to settle 30 seconds before using it. If the sensor isn't behaving as expected, check the two orange potentiometers on the side:

- Sensitivity (SENS): Adjusts the detection distance (usually 3 to 7 meters). Turning it clockwise increases sensitivity.

- Time Delay (TIME): Adjusts how long the output stays "HIGH" after motion is detected (usually 3 seconds to 5 minutes).

- The Jumper: Most HC-SR501 modules have a yellow jumper.

  - L (Single Trigger): The sensor resets the timer after a detection; it won't trigger again until the time delay ends.

  - H (Repeatable Trigger): Every time motion is detected, the timer restarts, keeping the output HIGH as long as there is continuous movement.

# Adding the Display to the Project

A single LED is fine, but you can also use the display to indicate when motion is detected. Let’s add the display to the project.

#include <Adafruit_GFX.h>

#include <Adafruit_ST7789.h>

#include <SPI.h>

// Pin definitions for ESP32-S3 (Adjust if using different pins)

#define TFT_CS         0

#define TFT_RST        1

#define TFT_DC        45

#define TFT_MOSI       3

#define TFT_SCLK      46

#define TFT_MISO      10

#define PIR_PIN       13

Adafruit_ST7789 tft = Adafruit_ST7789(TFT_CS, TFT_DC, TFT_RST);

void setup() {

// put your setup code here, to run once:

pinMode(PIR_PIN, INPUT);  // Set PIR pin as input

SPI.begin(TFT_SCLK, TFT_MISO, TFT_MOSI);

// Initialize ST7789 240x320

tft.init(240, 320);

Serial.begin(115200);

tft.init(240, 320);

tft.setRotation(1); // Adjust rotation as needed

tft.fillScreen(ST77XX_BLACK);

tft.setCursor(60, 100);

tft.setTextColor(ST77XX_GREEN);

tft.setTextSize(3);

tft.println("Calibrating Sensor");

delay(30000);

tft.fillScreen(ST77XX_BLACK);

}

void loop() {

int motionState = digitalRead(PIR_PIN); // Read the sensor

tft.setCursor(60, 100);

tft.setTextColor(ST77XX_GREEN);

tft.setTextSize(3);

if (motionState) {

tft.println("Motion Detected!");

}

else

{

tft.println("No Motion Detected.");

}

delay(500);

tft.fillScreen(ST77XX_BLACK);

}

Now when the motion is sensed you’ll see the indication on the Display.

# Taking the Project Further

There are several ways to take the project further:

- Add relay control to control a motor, fan, or light.

- Add the capability to a web page to indicate motion remotely.